Work Paper SCE13LG046

Revision 2

Southern California Edison Company

Interior Pulse Start Metal

Halide (PSMH) Fixtures

# At-a-Glance Summary

| Applicable Measure Codes: | LT-50223, LT-38071, LT-71883, LT-64118, LT-18932, LT-48241, LT-10924, LT-21092, LT-32093, LT-87566, LT-27452 |
| --- | --- |
| Measure Description: | Pulse-start metal halide fixtures |
| Base Case Description: | Base case fixtures vary by measure and can be incandescent, metal halide, high pressure sodium, or mercury vapor fixtures of various wattages. |
| Energy Impact Common Units: | Fixture |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | EUL: (70,000 hours)/(Annual hours of operation) or 15 years, whichever is less RUL: (70,000 hours or 4,000 hours)/(Annual hours of operation)/3 |
| Measure Application Type: | Replace on Burnout (ROB) Retrofit (RET) |
| Net-to-Gross Ratios: | Direct Install: 0.6 Direct Install / Hard-to-Reach: 0.85 Direct Install / Non-Hard-to-Reach: 0.6 |
| Important Comments: | - Measures LT-18932, LT-38071, LT-48241, LT-50223, LT-64118, and LT-71883 will be sunsetted beginning in the 2015 program year. They will remain active through the end of the 2013-14 program cycle. - This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG046.0 | No | 5/25/12 | Mike Green/Lincus, Brian V. O’Keefe/SCE | Updated to 2013-14 work paper template. |
| SCE13LG046.1 | Yes | 8/5/13 | Jason Wang/SCE | Updated per 3/1/13 ED Workpaper Disposition for Lighting Retrofits. |
| SCE15LG046.2 | Yes | 4/16/14 | Siobhan McCabe/TRC Energy Services | - Updated to new template - Updated per 12/14/13 ED Workpaper Disposition for Lighting Retrofits. |
|  | No | 6/12/14 | Jason Wang/SCE | - Work paper updated for reporting period, effective 7/1/2014 – 12/31/14. - Used base, code, and measure wattages, and measure IDs specified in the May 30 ED disposition, unless otherwise specified. - Added Direct Install non-hard-to-reach with 0.6 NTG in the Cost NTG Matrix |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

The measure case is a Pulse Start Metal Halide (PSMH) fixture. The base case is a High Intensity Discharge (HID) fixture, which can be Mercury Vapor, Metal Halide, High-pressure Sodium or Incandescent. Table 1 indicates the specific wattage requirements for each measure.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-50223 | Up to 70 Watt Pulse Start HID replacing less than 100 Watt lamp base case |
| LT-38071 | Up to 125 Watt Pulse Start HID replacing 101 - 175 Watt lamp base case |
| LT-71883 | Up to 175 Watt Pulse Start HID replacing 176 - 399 Watt lamp base case |
| LT-64118 | Up to 250 Watt Pulse Start HID replacing 400 Watt lamp base case |
| LT-18932 | Up to 600 Watt Tier 1 Pulse Start HID replacing greater than 400 Watt lamp base case |
| LT-48241 | Up to 750 Watt Tier 2 Pulse Start HID replacing greater than 400 Watt lamp base case |
| LT-10924 | 32 Watt Metal Halide High Intensity Discharge replacing 100 Watt Incandescent |
| LT-21092 | 175 Watt Pulse Start HID replacing 250 Watt Metal Halide |
| LT-32093 | 175 Watt Pulse Start HID replacing 500 Watt Incandescent |
| LT-87566 | 250 Watt Pulse Start HID replacing 400 Watt Mercury Vapor |
| LT-27452 | 350 Watt Pulse Start HID replacing 1000 Watt Incandescent |

Eligibility Requirements

- Measure case fixtures must have PSMH lamps and electronic ballasts.

- All non-residential building types and all SCE climate zones are eligible.

Implementation Requirements

- Measure case fixtures must replace base case fixtures one-for-one.

- Only interior installations are eligible; exterior installations do not qualify.

- All replacements must be hardwired.

- If the base case wattage is 400 Watt or greater, the replacement fixtures must be installed at a height greater than 12’ above the finished floor.

## 1.2 Technical Description

High Intensity Discharge light sources include metal halide (MH), mercury vapor (MV), high pressure sodium (HPS), and low pressure sodium (LPS). An HID lamp contains 2 tungsten electrodes located at opposite ends of a sealed arc tube containing various gases and metal halides. When high voltage is applied to the electrodes, an arc is created between them, producing light. HID fixtures are highly efficacious, have long lives and are excellent point sources with good control. Ballasts are required to drive HID lamps.

Pulse-start vs probe-star t metal halide

A traditional probe-start MH fixture uses a third electrode (starter electrode) to initiate the arc. A pulse-start metal halide (PSMH) fixture uses an ignitor that provides high voltage pulses to initiate the arc. When compared to MH fixtures, PSMH fixtures have a shorter warm-up time, longer lamp life, and higher lumen maintenance (ability to maintain light output over time).

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this workpaper and application types used by READi.

For the measures with specified wattage ranges (LT-50223, LT-38071, LT-71883, LT-64118, LT-18932, and LT-48241):

- The install type is Replace on Burnout (ROB)

- The delivery methods are:

  - Financial Support / Down-Stream Incentive – Deemed

  - Partnership / Down-Stream Incentive – Deemed

  - Midstream Programs / Mid-Stream Incentive

For the measures with specified wattage values (LT-10924, LT-21092, LT-32093, LT-87566, and LT-27452):

- The install type is Retrofit (RET).

- The delivery methods are:

- Financial Support / Direct Install

- Financial Support / Direct Install / Hard-to-Reach

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

These measures are available from DEER when using the READi tool to access the “Ex-Ante 13-14 Cycle” database, but the latest interactive effects from February 2015 have not been implemented there yet. Therefore, the calculation template (Attachment 1) was used to calculate savings.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | N/A |
| DEER Version | DEER14 |
| DEER Run ID and Measure Name (Sample) | C-In-PSMH-70w(79w)-dWP49 |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGRs for these measures are shown in Table 3 below.

The Energy Division Workpaper Disposition for Lighting Retrofits [385] recommends an NTG ratio of 0.6 for all delivery methods. SCE will use a 0.85 ratio for Direct Install / Hard-to-Reach but will use 0.6 for Direct Install / Non-Hard-to-Reach.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.60 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | Any | 0.60 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | Any | 0.60 |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Com | Any | DirInstall | 0.85 |
| Ind-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ind | Any | DirInstall | 0.85 |
| Agricult-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ag | Any | DirInstall | 0.85 |
| Com-Default-HTR-di | ED Disposition: All other EEM with no evaluated NTGR; direct install only. | Com | Any | DirInstall | 0.60 |
| Ind-Default-HTR-di | ED Disposition: All other EEM with no evaluated NTGR; direct install only. | Ind | Any | DirInstall | 0.60 |
| Agricult-Default-HTR-di | ED Disposition: All other EEM with no evaluated NTGR; direct install only. | Ag | Any | DirInstall | 0.60 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the non-residential defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no final version of the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

The Energy Division Workpaper Disposition for Lighting Retrofits (Attachment 3, [385]) recommends an IR of 1.0 for all delivery methods.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Com-HID-SCE-DI | 2013-2014_LightingRetrofit_Disposition-30May2014; GSIA Ref for SCE13LG046 Pulse Start HID - Direct Install | Any | Any | DirInstall | 0.95 |
| Com-HID-SCE-DSD | 2013-2014_LightingRetrofit_Disposition-30May2014; GSIA Ref for SCE13LG046 Pulse Start HID - Downstream | Any | Any | PreRebDown | 0.93 |
| Com-HID-SCE-MS | 2013-2014_LightingRetrofit_Disposition-30May2014; GSIA Ref for SCE13LG046 Pulse Start HID - Midstream | Any | Any | Mid-Stream Incentive | 1.0 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measure Case UseCategory | Lighting |
| Measure Case UseSubCats | Indoor Lighting |
| Measure Case TechGroups | Lighting – Hard-wired fixtures (Ltg_Wired) |
| Measure Case TechTypes | HID Lamp with Ballast (HID_LmpBlst) |
| Base Case TechGroups | Lighting – Hard-wired fixtures (Ltg_Wired) |
| Base Case TechTypes | HID Lamp with Ballast (HID_LmpBlst) |

### 1.4.2 Codes and Standards Analysis

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | Section 141.0(b), 140.6(c)2 | July 1, 2014 |
| Title 20 (2014) | Section 1605.3(n) | July 1, 2014 |

Title 20 2012 [356]:

Section 1605.3(n) provides standards for metal halides:

Title 24 2013 [355]: Lighting System Alterations and Modifications-in-Place will trigger Title 24; see requirements below:

Section 140.6(c)2, Area Category Method, requires that lighting power density be calculated for each retrofit. Refer to Table 140.6-B for the allowed lighting power density by building type and Table 140.6-C for the allowed values by space type.

### 1.4.3 Non-DEER Study Review

No non-DEER studies are referenced in this workpaper.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2013-14 program cycle on www.deeresources.com. The DEER documentation “DEER2014-EUL-table-update_2014-02-05.xlsx” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

In the EUL/RUL Values and Summary Documentation, provided on the DEER website, the effective useful life (EUL) for Metal Halide fixtures is calculated by dividing a 70,000 lifetime operating hour value by the annual building operating hours [49] (use occupancy sensor hours for non-residential measures). If this value exceeds 15 years, then 15 years is used.

For RET measures, the RUL ID depends on the measure. Metal halide and PSMH baselines use the RUL ID “Iltg-MH.” Incandescent baselines use the RUL ID “Iltg-Com-Incand” with a lifetime of 4,000 hours. The RUL is 1/3 of the RUL ID value. The EUL-RUL is the EUL ID value minus the RUL.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Building Type | EUL (Years) | RUL ID | RUL (Years) |
| --- | --- | --- | --- | --- |
| ILtg-MH | Various | 70,000 / Annual OS Operating Hours | ILtg-MH | (70,000 / Annual Non-OS Operating Hours) / 3 |
|  |  |  | ILtg-Com-Incand | (4,000 / Annual Non-OS Operating Hours) / 3 |

# Section 2. Energy Savings & Demand Reduction Calculations

Table 8 contains the assumed base, code, and measure case wattages, from the Energy Division May 30 Lighting Disposition (Attachment 3) and other sources.

Table 8  Base, Code, and Measure Wattages

| Solution Code | Measure Name | Install Type | Customer Existing Wattage | Code Case Wattage | Measure Case Wattage |
| --- | --- | --- | --- | --- | --- |
| LT-50223 | Up to 70 Watt Pulse Start HID replacing less than 100 Watt lamp base case | ROB | N/A | 128 | 92* |
| LT-38071 | Up to 125 Watt Pulse Start HID replacing 101 - 175 Watt lamp base case | ROB | N/A | 208** | 150 |
| LT-71883 | Up to 175 Watt Pulse Start HID replacing 176 - 399 Watt lamp base case | ROB | N/A | 232 | 208* |
| LT-64118 | Up to 250 Watt Pulse Start HID replacing 400 Watt lamp base case | ROB | N/A | 400** | 288* |
| LT-18932 | Up to 600 Watt Tier 1 Pulse Start HID replacing greater than 400 Watt lamp base case | ROB | N/A | 940** | 506* |
| LT-48241 | Up to 750 Watt Tier 2 Pulse Start HID replacing greater than 400 Watt lamp base case | ROB | N/A | 1080 | 818* |
| LT-10924 | 32 Watt Metal Halide High Intensity Discharge replacing 100 Watt Incandescent | RET | 100 | 43 | 43 |
| LT-21092 | 175 Watt Pulse Start HID replacing 250 Watt Metal Halide | RET | 295 | 232 | 208 |
| LT-32093 | 175 Watt Pulse Start HID replacing 500 Watt Incandescent | RET | 500 | 208 | 208 |
| LT-87566 | 250 Watt Pulse Start HID replacing 400 Watt Mercury Vapor | RET | 455 | 288 | 288 |
| LT-27452 | 350 Watt Pulse Start HID replacing 1000 Watt Incandescent | RET | 1000 | 400 | 400 |

*This value comes from the previous revision of this work paper (Revision 1) and differs from the May 30 disposition value. Since the disposition document only addresses code baseline and not the measure case, it is unclear why the disposition’s recommended measure case wattages differ from those in Revision 1 of this work paper. Therefore, the disposition value is not used.

**This is the wattage for a PSMH fixture that is lumen equivalent to the measure case fixture. This value differs from the May 30 disposition value.

Note that for RET measures where the code and measure wattages are the same, the 2nd baseline savings will be zero.

Sample Savings Calculation: 175 Watt Pulse Start HID replacing 250 Watt Metal Halide, Assembly, CZ 6

1st Baseline kWh Savings

=PrekW*NonOS Hours-MeasurekW*OS hours*Interactive Effects

=295 W*2609.6 hours-208*2531.51000*1.126

=273.93 kWh

1st Baseline kW Reduction

=PrekW*NonOS CDF-MeasurekW*OS CDF*Interactive Effects

=295W*0.53187-208*0.519111000*1.2389

=0.06062 kW

2nd Baseline kWh Savings

=(CodekW-MeasurekW)*OShours*Interactive Effects

=232-2081000kW*2531.5 hours*1.126

=68.41 kWh

2nd Baseline kW Reduction

=CodekW-MeasurekW*OS CDF*Interactive Effects

=232-2081000kW*0.51911*1.2389

=0.01544 kW

A complete list of savings is in Attachment 1.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DEER:Indoor_Non-CFL_Ltg load shape.  See Table 9 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 9 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Agricultural | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Assembly | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Primary School | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Secondary School | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Relocatable Classroom | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Community College | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - University | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Grocery | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Food Store | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Health/Medical - Hospital | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Health/Medical - Nursing Home | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Health/Medical - Clinic | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Lodging - Hotel | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Lodging - Guest Rooms | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Lodging - Motel | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Manufacturing - Bio/Tech | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Manufacturing - Light Industrial | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Industrial | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Misc - Commercial | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Office – Large | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Office – Small | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Restaurant - Fast-Food | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Restaurant - Sit-Down | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Retail - Multistory Large | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Retail – Single-Story Large | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Retail – Small | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Storage - Conditioned | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Storage - Unconditioned | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Transportation - Communication – Utilities | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Warehouse – Refrigerated | NON_RES | DEER:Indoor_Non-CFL_Ltg |

# Section 4. Base Case & Measure Costs

Direct Install measures use program tracking systems for measure costs.

## 4.1 Base Case Cost

Base case costs are from DEER08 [215] and retailers. Cost calculations are in the cost spreadsheet (Attachment 2).

Table 10 Base Case Costs

| Measure Name | Base Case Equipment Source | Base Case Equipment Cost | Base Case Labor Source | Base Case Labor Cost | Total Base Case Cost |
| --- | --- | --- | --- | --- | --- |
| Up to 70 Watt Pulse Start HID replacing less than 100 Watt lamp base case | Retailers | $75.53 | DEER08: 250wMHMgC295w-Rpl | $72.17 | $147.70 |
| Up to 125 Watt Pulse Start HID replacing 101 - 175 Watt lamp base case | Retailers | $87.78 | DEER08: 250wMHMgC295w-Rpl | $72.17 | $159.95 |
| Up to 175 Watt Pulse Start HID replacing 176 - 399 Watt lamp base case | Retailers | $106.16 | DEER08: 250wMHMgC295w-Rpl | $72.17 | $178.33 |
| Up to 250 Watt Pulse Start HID replacing 400 Watt lamp base case | Retailers | $120.54 | DEER08: 400wMVMgC455w-Rpl | $72.17 | $192.71 |
| Up to 600 Watt Tier 1 Pulse Start HID replacing greater than 400 Watt lamp base case | Retailers | $226.76 | DEER08: 400wMVMgC455w-Rpl | $72.17 | $298.94 |
| Up to 750 Watt Tier 2 Pulse Start HID replacing greater than 400 Watt lamp base case | Retailers | $226.76 | DEER08: 400wMVMgC455w-Rpl | $72.17 | $298.94 |
| 32 Watt Metal Halide High Intensity Discharge replacing 100 Watt Incandescent | DEER08: D08-NE-ILtg-Inc-100W | $0.68 | DEER08: 100wInc100w-Rpl | $18.04 | $18.72 |
| 175 Watt Pulse Start HID replacing 250 Watt Metal Halide | DEER08: 250wMHMgC295w-Rpl | $152.84 | DEER08: 250wMHMgC295w-Rpl | $72.17 | $225.01 |
| 175 Watt Pulse Start HID replacing 500 Watt Incandescent | Retailers | $8.98 | DEER08: 500wInc500w-Rpl | $72.17 | $81.15 |
| 250 Watt Pulse Start HID replacing 400 Watt Mercury Vapor | Retailers | $79.26 | DEER08: 400wMVMgC455w-Rpl | $72.17 | $151.43 |
| 350 Watt Pulse Start HID replacing 1000 Watt Incandescent | Retailers | $33.10 | DEER08: 1kwInc1000w-Rpl | $72.17 | $105.27 |

## 4.2 Measure Case Cost

Measure case costs are from DEER08, vendors, and RS Means.

Table 11 Measure Case Costs

| Measure Name | Measure Case Source | Measure Equipment Cost | Measure Labor Cost | Total Measure Cost |
| --- | --- | --- | --- | --- |
| Up to 70 Watt Pulse Start HID replacing less than 100 Watt lamp base case | Vendor/RS Means | $122.56 | $72.17 | $194.73 |
| Up to 125 Watt Pulse Start HID replacing 101 - 175 Watt lamp base case | Vendor/RS Means | $112.97 | $72.17 | $185.14 |
| Up to 175 Watt Pulse Start HID replacing 176 - 399 Watt lamp base case | Vendor/RS Means | $153.24 | $72.17 | $225.41 |
| Up to 250 Watt Pulse Start HID replacing 400 Watt lamp base case | Vendor/RS Means | $143.07 | $72.17 | $215.24 |
| Up to 600 Watt Tier 1 Pulse Start HID replacing greater than 400 Watt lamp base case | Vendor/RS Means | $143.07 | $72.17 | $215.24 |
| Up to 750 Watt Tier 2 Pulse Start HID replacing greater than 400 Watt lamp base case | Vendor/RS Means | $250.03 | $72.17 | $322.20 |
| 32 Watt Metal Halide High Intensity Discharge replacing 100 Watt Incandescent | DEER08: 32wMHR43w-Rpl | $111.69 | $18.04 | $129.73 |
| 175 Watt Pulse Start HID replacing 250 Watt Metal Halide | DEER08: 175wPSMHMgC208w-Rpl | $135.34 | $72.17 | $207.51 |
| 175 Watt Pulse Start HID replacing 500 Watt Incandescent | DEER08: 175wPSMHMgC208w-Rpl | $135.34 | $72.17 | $207.51 |
| 250 Watt Pulse Start HID replacing 400 Watt Mercury Vapor | DEER08: 250wPSMHMgC288w-Rpl | $148.33 | $72.17 | $220.50 |
| 350 Watt Pulse Start HID replacing 1000 Watt Incandescent | DEER08: 350wPSMHMgC400w-Rtr | $165.64 | $72.17 | $237.81 |

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For ROB, Gross Measure Cost (GMC) is the cost premium required to install an energy efficient measure over a less efficient piece of equipment. GMC is represented by the equation below:

GMC	= (Measure Equipment Cost + Measure Labor Cost)

– (Base Case Equipment Cost + Base Case Labor Cost)

### The equipment being replaced is assumed to have failed in place or is past its useful life.  In this scenario the customer is in the situation of having to purchase new equipment. The customer is faced with either purchasing standard efficiency or code baseline equipment versus energy efficient equipment. Because the customer will be spending money to replace their equipment anyway, the gross cost for the energy efficient measure is the premium paid above the non-efficient or code baseline equipment.

For RET, GMC is the full measure cost:

GMC	= (Measure Equipment Cost + Measure Labor Cost)

Table 12 Gross Measure Cost

| Measure Name | Total Base Case Cost | Total Measure Cost | Gross Measure Cost | Incremental Measure Cost |
| --- | --- | --- | --- | --- |
| Up to 70 Watt Pulse Start HID replacing less than 100 Watt lamp base case | $147.70 | $194.73 | $47.03 | $47.03 |
| Up to 125 Watt Pulse Start HID replacing 101 - 175 Watt lamp base case | $159.95 | $185.14 | $25.19 | $25.19 |
| Up to 175 Watt Pulse Start HID replacing 176 - 399 Watt lamp base case | $178.33 | $225.41 | $47.08 | $47.08 |
| Up to 250 Watt Pulse Start HID replacing 400 Watt lamp base case | $192.71 | $215.24 | $22.53 | $22.53 |
| Up to 600 Watt Tier 1 Pulse Start HID replacing greater than 400 Watt lamp base case | $298.94 | $215.24 | -$83.69 | -$83.69 |
| Up to 750 Watt Tier 2 Pulse Start HID replacing greater than 400 Watt lamp base case | $298.94 | $322.20 | $23.27 | $23.27 |
| 32 Watt Metal Halide High Intensity Discharge replacing 100 Watt Incandescent | $18.72 | $129.73 | $129.73 | $111.01 |
| 175 Watt Pulse Start HID replacing 250 Watt Metal Halide | $225.01 | $207.51 | $207.51 | -$17.50 |
| 175 Watt Pulse Start HID replacing 500 Watt Incandescent | $81.15 | $207.51 | $207.51 | $126.36 |
| 250 Watt Pulse Start HID replacing 400 Watt Mercury Vapor | $151.43 | $220.50 | $220.50 | $69.07 |
| 350 Watt Pulse Start HID replacing 1000 Watt Incandescent | $105.27 | $237.81 | $237.81 | $132.54 |

### 4.3.2 Incremental Measure Cost

For ROB, Incremental Measure Cost (IMC) is the same as the GMC.

For REF, IMC is:

IMC	= (Measure Equipment Cost + Measure Labor Cost)

– (Base Case Equipment Cost + Base Case Labor Cost)

Refer to Table 12 for the GMC and IMC.

# Attachments

# References

| [31] | Load Shape Update Initiative - KEMA / JJ Hirsch and Assoc. / Itron Inc. - November 17, 2006 |
| --- | --- |
| [49] | DEER Database |
| [215] | Revised DEER Measure Cost Summary |
| [351] | Energy Efficiency Policy Manual-Version 5 |
| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |
| [356] | 2012 Appliance Efficiency Regulations (Title 20) |
| [385] | Workpaper Disposition for Lighting Retrofits |

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |